SD6G Bulldozer

| 发动机 Engine |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 型号 Model |  |  |  |  | 上柴 C6121ZG72B |  |  |  |  |  |
| 形式 Type |  |  |  |  | 水冷、单行、立式、四冲程、涡轮增压 |  |  |  |  |  |
|  |  |  |  |  | water-cooled, single-row, vertical, 4-stroke-cycle, turbo-charged |  |  |  |  |  |
| 额定转速 Rated revolution (rpm) |  |  |  |  | 1900rpm |  |  |  |  |  |
| 额定功率 Rated power (KW) |  |  |  |  | 131kw |  |  |  |  |  |
| 燃油消耗率 Rated fuel consumption (g/kw.h) |  |  |  |  | 215 |  |  |  |  |  |
| 最大扭矩 Max. torque (N.m/rpm) |  |  |  |  | 770/1400 |  |  |  |  |  |
| 启动方式 Starting Mode |  |  |  |  | 24伏电起动 24V electric starting |  |  |  |  |  |
| 整机性能参数 Main Specification |  |  |  |  |  |  |  |  |  |  |
| 使用重量(不带松土器) Operating Weight (kg) |  |  |  |  | 16200kg |  |  |  |  |  |
| 松土器质量 Ripper weight (kg) |  |  |  |  | 1600kg |  |  |  |  |  |
| 接地压力（不带松土器）(kPa) |  |  |  |  | 55.23kPa |  |  |  |  |  |
| 最小离地间隙 (mm) |  |  |  |  | 445mm |  |  |  |  |  |
| 爬坡性能 Grade ability (0) |  |  |  |  | 300 |  |  |  |  |  |
| 履带接地长度 (mm) |  |  |  |  | 2667 |  |  |  |  |  |
| 履带中心距 (mm) |  |  |  |  | 1880mm |  |  |  |  |  |
| 最大牵引力 |  |  |  |  | 145KN |  |  |  |  |  |
| 生产率(运距40米) Dozing operation rate(moving distance 40m) |  |  |  |  | 320m3/h |  |  |  |  |  |
| 理论行使速度 Theory travel speed (km/h) |  |  |  |  |  |  |  |  |  |  |
| 前进一档 1st gear forward 0-4.0 |  |  |  |  | 后退一档 1st gear backward 0-4.8 |  |  |  |  |  |
| 前进二档 2nd gear forward 0-6.9 |  |  |  |  | 后退二档 2nd gear backward 0-8.4 |  |  |  |  |  |
| 前进三档 3rd gear forward 0-10.9 |  |  |  |  | 后退三档 3rd gear backward 0-12.9 |  |  |  |  |  |
| 传动系统 Power transmitting system |  |  |  |  |  |  |  |  |  |  |
| 液力变矩器 Torque converter |  |  |  |  | 单级单相三元件 Single-stage, single-phase, 3-element |  |  |  |  |  |
| 变速器箱 Transmission |  |  |  |  | 行星齿轮，多片离合器，液压结合器强制润滑式 Planetary gear, multi-disc clutch, hydraulic engaged, forced lubrication |  |  |  |  |  |
| 中央传动 Main drive |  |  |  |  | 螺旋锥齿轮，一级减速，飞溅润滑 Spiral bevel gear, single-stage speed reduction, splash lubrication |  |  |  |  |  |
| 转向离合器 Steering clutch |  |  |  |  | 湿式、多片、弹簧压紧、液压分离、液压操作 Wet, multi-disc, spring loaded, hydraulically separated, hydraulic control |  |  |  |  |  |
| 转向制动器 Steering brake |  |  |  |  | 湿式、浮动、双向带式，脚踏式机械操纵 Oil-cooled, hydraulically-boosted two direction contracting band brakes with foot pedal control |  |  |  |  |  |
| 最终传动 Final drive |  |  |  |  | 二级直齿轮减速 Double spur gear reduction |  |  |  |  |  |
| 行走系统 Undercarriage system |  |  |  |  |  |  |  |  |  |  |
| 形式 Type |  |  |  |  | 八字梁摆动式，平衡梁悬挂结构 |  |  |  |  |  |
|  |  |  |  |  | Swing type of sprayed beam, suspended structure of equalizer bar |  |  |  |  |  |
| 托轮数（单侧） Number of carrier rollers (each side) |  |  |  |  | 2 |  |  |  |  |  |
| 支重轮数（单侧） Number of track rollers (each side) |  |  |  |  | 7 |  |  |  |  |  |
| 履带形式 Track type |  |  |  |  | 装配式单履齿（每边39块） Assembled single-grouser shoes(39/each side) |  |  |  |  |  |
| 履带板宽度(标准) Width of track shoes（Standard） |  |  |  |  | 560mm |  |  |  |  |  |
| 链轨节距 Pitch |  |  |  |  | 203mm |  |  |  |  |  |
| 工作装置液压系统 Implement hydraulic system |  |  |  |  |  |  |  |  |  |  |
| 最高工作油压 Max pressure |  |  |  |  | 15.5Mpa |  |  |  |  |  |
| 工作油泵形式 Pump type |  |  |  |  | 双联齿轮泵 Double gear pump |  |  |  |  |  |
| 流量 Discharge |  |  |  |  | 178升/分钟 178L/min |  |  |  |  |  |
| 操作阀形式 Control valve type |  |  |  |  | 滑阀式 Sliding valve |  |  |  |  |  |
| 工作油缸内径 X 数量 Bore of the working cylinder X number |  |  |  |  | 100mm x 1 |  |  |  |  |  |
| 倾斜缸内径 X 数量 Bore X NO. (Tilting cylinder) |  |  |  |  | 1500mm x 1 |  |  |  |  |  |
| 防翻防落物保护配空调司机棚 ROPS/FOPS with air conditioner cab |  |  |  |  |  |  |  |  |  |  |
| 司机棚为六边形薄壁箱形结构。六面均装有玻璃，视野开阔，后面是推拉玻璃。空调为冷热两种功能。防翻防落物防护架结构刚性强，推土机在非常状态时能有效的保护司机的安全。 |  |  |  |  | The cab is hexagonal thin wall box-type structure. Glass is mounted at six sides with wide view area. Pulling and pushing glass is installed at back side. Air conditioner adapts a type of cooling and heating. The structure of Rop/FOPS is rigid. It can protect the driver efficiently when the bulldozer is in special condition. |  |  |  |  |  |
| 工作装置 Working equipment |  |  |  |  |  |  |  |  |  |  |
| 铲刀形式 | 履带接地长度A | 整机长度B | 铲刀高度C | 整机高度 D |  | 铲刀宽度E | 履带接地中心距F | 挖掘深度 | 提升高度 | 铲刀容量 |
| 直倾铲 | 2667 | 6140 | 1170 | 3077 |  | 3270 | 1880 | 592 | 998 | 4.5 |
| 推煤铲 |  |  |  |  |  |  |  |  |  |  |
| 角 铲 | 2667 | 5156 | 1140 | 3077 |  | 3762 | 1880 | 510 | 1040 | 4.3 |
| 低比压铲 |  |  |  |  |  |  |  |  |  |  |

| Blade Type | Track ground contact length A | Length of machinery B | Height of blade C | Height of machinery D | Width of blade E | Track center distance F | Digging depth | Lifting height | Blade capacity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Straight tilting blade | 2667 | 6140 | 1170 | 3077 | 3270 | 1880 | 592 | 998 | 4.5 |
| Coal pushing blade |  |  |  |  |  |  |  |  |  |
| Angle blade | 2667 | 5156 | 1140 | 3077 | 3762 | 1880 | 510 | 1040 | 4.3 |
| LGP blade |  |  |  |  |  |  |  |  |  |