SD7推土机Bulldozer

| 发动机 Engine |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 型号 Model |  |  |  |  | Cummins NT855-C280S10 |  |  |  |  |  |
| 形式 Type |  |  |  |  | 水冷单行、立式、四冲程、涡轮增压、六缸，140mm缸径 |  |  |  |  |  |
|  |  |  |  |  | water-cooled, single line, vertical, four-stroke, turbocharged diesel, 6-cylinder, diameter: A.140mm B.121mm |  |  |  |  |  |
| 额定转速 Rated revolution (rpm) |  |  |  |  | 2100rpm |  |  |  |  |  |
| 额定功率 Rated power (KW) |  |  |  |  | 169kw |  |  |  |  |  |
| 燃油消耗率 Rated fuel consumption (g/kw.h) |  |  |  |  | 235 |  |  |  |  |  |
| 最大扭矩 Max. torque (N.m/rpm) |  |  |  |  | 1000/1450 |  |  |  |  |  |
| 启动方式 Starting Mode |  |  |  |  | 24伏电起动 24V electric starting |  |  |  |  |  |
| 整机性能参数 Main Specification |  |  |  |  |  |  |  |  |  |  |
| 使用重量(不带松土器) Operating Weight (kg) |  |  |  |  | 23800Kg |  |  |  |  |  |
| 松土器质量 Ripper weight (kg) |  |  |  |  | 3900Kg |  |  |  |  |  |
| 接地压力（不带松土器）(kPa) |  |  |  |  | 0.0719Mpa |  |  |  |  |  |
| 最小离地间隙 (mm) |  |  |  |  | 404mm |  |  |  |  |  |
| 爬坡性能 Grade ability (0) |  |  |  |  | 300 |  |  |  |  |  |
| 履带接地长度 (mm) |  |  |  |  | 2890mm |  |  |  |  |  |
| 履带中心距 (mm) |  |  |  |  | 1980mm |  |  |  |  |  |
| 最大牵引力 (kN) |  |  |  |  | 280 |  |  |  |  |  |
| 生产率(运距40米) Dozing operation rate(moving distance 40m) |  |  |  |  | 350m3/h |  |  |  |  |  |
| 理论行使速度 Theory travel speed (km/h) |  |  |  |  |  |  |  |  |  |  |
| 前进一档 1st gear forward 0-3.9 |  |  |  |  | 后退一档 1st gear backward 0-4.8 |  |  |  |  |  |
| 前进二档 2nd gear forward 0-6.5 |  |  |  |  | 后退二档 2nd gear backward 0-8.2 |  |  |  |  |  |
| 前进三档 3rd gear forward 0-10.9 |  |  |  |  | 后退三档 3rd gear backward 0-13.2 |  |  |  |  |  |
| 传动系统 Power transmitting system |  |  |  |  |  |  |  |  |  |  |
| 液力变矩器 Torque converter |  |  |  |  | 单级单相三元件 Single-stage, single-phase, 3-element |  |  |  |  |  |
| 变速器箱 Transmission |  |  |  |  | 行星齿轮，多片离合器，液压结合器强制润滑式 Planetary gear, multi-disc clutch, hydraulic engaged, forced lubrication |  |  |  |  |  |
| 中央传动 Main drive |  |  |  |  | 螺旋锥齿轮，一级减速，飞溅润滑 Spiral bevel gear, single-stage speed reduction, splash lubrication |  |  |  |  |  |
| 转向离合器 Steering clutch |  |  |  |  | 湿式、多片、液压分离、液压操作 Wet, multi-disc, spring loaded, hydraulically separated, hydraulic control |  |  |  |  |  |
| 转向制动器 |  |  |  |  | 湿式、多片、弹簧压紧、液压分离 Wet, multi-disc, spring loaded, hydraulically separated, |  |  |  |  |  |
| 最终传动 Final drive |  |  |  |  | 二级直齿轮减速，飞溅润滑 Two-stage speed reduction of spur gear, splash lubrication |  |  |  |  |  |
| 行走系统 Undercarriage system |  |  |  |  |  |  |  |  |  |  |
| 形式 Type |  |  |  |  | 八字梁摆动式，平衡梁悬挂结构 |  |  |  |  |  |
|  |  |  |  |  | Swing type of sprayed beam, suspended structure of equalizer bar |  |  |  |  |  |
| 托轮数（单侧） Number of carrier rollers (each side) |  |  |  |  | 2 |  |  |  |  |  |
| 支重轮数（单侧） Number of track rollers (each side) |  |  |  |  | 7个/每边（单边5个双边2个） 7/each side(5 single, 2 double) |  |  |  |  |  |
| 履带形式 Track type |  |  |  |  | 装配式单履齿（每边39块） Assembled single-grouser shoes(39/each side) |  |  |  |  |  |
| 履带板宽度(标准) Width of track shoes（Standard） |  |  |  |  | 560mm |  |  |  |  |  |
| 链轨节距 Pitch |  |  |  |  | 216mm |  |  |  |  |  |
| 工作装置液压系统 Implement hydraulic system |  |  |  |  |  |  |  |  |  |  |
| 最高工作油压 Max pressure |  |  |  |  | 19.1Mpa(195kg/cm2) |  |  |  |  |  |
| 工作油泵形式 Pump type |  |  |  |  | 双联齿轮泵 Double gear pump |  |  |  |  |  |
| 流量 Discharge |  |  |  |  | 194升/分钟（发动机转速2000rpm） 194L/min(at engine speed of 2000rpm) |  |  |  |  |  |
| 操作阀形式 Control valve type |  |  |  |  | 滑阀式 Sliding valve |  |  |  |  |  |
| 工作油缸内径 X 数量 Bore of the working cylinder X number |  |  |  |  | 160mm x 1 |  |  |  |  |  |
| 倾斜缸内径 X 数量 Bore X NO. (Tilting cylinder) |  |  |  |  | 60mm x 1 |  |  |  |  |  |
| 防倾翻保护配空调司机棚 ROPS with air conditioner cab |  |  |  |  |  |  |  |  |  |  |
| 司机棚为六边形薄壁箱形结构。六面均装有玻璃，视野开阔，后面是推拉玻璃。空调为冷热两种功能。翻车防护架结构刚性强，推土机在非常状态时能有效的保护司机的安全。 |  |  |  |  | The cab is hexagonal thin wall box-type structure. Glass is mounted at six sides with wide view area. Pulling and pushing glass is installed at back side. Air conditioner adapts a type of cooling and heating. The structure of Rop is rigid. It can protect the driver efficiently when the bulldozer is in special condition. |  |  |  |  |  |
| 工作装置 Working equipment |  |  |  |  |  |  |  |  |  |  |
| 铲刀形式 | 履带接地长度A | 整机长度B | 铲刀高度C | 整机高度 D |  | 铲刀宽度E | 履带接地中心距F | 挖掘深度 | 提升高度 | 铲刀容量 |
| 直倾铲 | 2895 | 7604 | 1550 | 3402 |  | 3500 | 1980 | 500 | 1170 | 8.4 |
| 推煤铲 | 2895 | 7880 | 1350 | 3402 |  | 4428 | 1980 | 670 | 1115 | 8.1 |
| 角 铲 | 2895 | 7571 | 1100 | 3402 |  | 4523 | 1980 | 670 | 1115 | 5.2 |
| 低比压铲 | 3185 | 7190 | 1150 | 3482 |  | 4382 | 2235 | 635 | 1170 | 5.8 |

| Blade Type | Track ground contact length A | Length of machinery B | Height of blade C | Height of machinery D | Width of blade E | Track center distance F | Digging depth | Lifting height | Blade capacity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Straight tilting blade | 2895 | 7604 | 1550 | 3402 | 3500 | 1980 | 500 | 1170 | 8.4 |
| Coal pushing blade | 2895 | 7880 | 1350 | 3402 | 4428 | 1980 | 670 | 1115 | 8.1 |
| Angle blade | 2895 | 7571 | 1100 | 3402 | 4523 | 1980 | 670 | 1115 | 5.2 |
| LGP blade | 3185 | 7190 | 1150 | 3482 | 4382 | 2235 | 635 | 1170 | 5.8 |