TY165-2推土机Bulldozer

| 发动机 Engine |  |
| --- | --- |
| 型号 Model | 潍柴 WD10G178E15 |
| 形式 Type | 水冷、单行、立式、四冲程、涡轮增压 |
|  | water-cooled, single-row, vertical, 4-stroke-cycle, turbo-charged |
| 额定转速 Rated revolution (rpm) | 1850rpm |
| 额定功率 Rated power (KW) | 131kw |
| 燃油消耗率 Rated fuel consumption (g/kw.h) | 244 |
| 最大扭矩 Max. torque (N.m/rpm) | 790/1400 |
| 启动方式 Starting Mode | 24伏电起动 24V electric starting |
| 整机性能参数 Main Specification |  |
| 使用重量(不带松土器) Operating Weight (kg) | 17800 |
| 松土器质量 Ripper weight (kg) | 1600 |
| 接地压力（不带松土器）(kPa) | 0.0692Mpa |
| 最小离地间隙 (mm) | 352.5mm |
| 爬坡性能 Grade ability (0) | 300 |
| 履带接地长度 (mm) | 2570 |
| 履带中心距 (mm) | 1880mm |
| 最大牵引力 | 139.6KN |
| 生产率(运距40米) Dozing operation rate(moving distance 40m) | 300m3/h |
| 理论行使速度 Theory travel speed (km/h) |  |
| 前进一档 1st gear forward 0-3.32 | 后退一档 1st gear backward 0-4.00 |
| 前进二档 2nd gear forward 0-6.26 | 后退二档 2nd gear backward 0-7.57 |
| 前进三档 3rd gear forward 0-11.40 | 后退三档 3rd gear backward 0-13.87 |
| 传动系统 Power transmitting system |  |
| 液力变矩器 Torque converter | 单级单相三元件 Single-stage, single-phase, 3-element |
| 变速器箱 Transmission | 行星齿轮，多片离合器，液压结合器强制润滑式 Planetary gear, multi-disc clutch, hydraulic engaged, forced lubrication |
| 中央传动 Main drive | 螺旋锥齿轮，一级减速，飞溅润滑 Spiral bevel gear, single-stage speed reduction, splash lubrication |
| 转向离合器 Steering clutch | 湿式、多片、弹簧压紧、液压分离、液压操作 Wet, multi-disc, spring loaded, hydraulically separated, hydraulic control |
| 转向制动器 Steering brake | 湿式、浮动、双向带式，脚踏式机械操纵 Brake is oil two direction floating band brake operated by mechanical foot pedal. |
| 最终传动 Final drive | 二级直齿轮减速，飞溅润滑 Two-stage speed reduction of spur gear, splash lubrication |
| 行走系统 Undercarriage system |  |
| 形式 Type | 八字梁摆动式，平衡梁悬挂结构 |
|  | Swing type of sprayed beam, suspended structure of equalizer bar |
| 托轮数（单侧） Number of carrier rollers (each side) | 2 |
| 支重轮数（单侧） Number of track rollers (each side) | 6 |
| 履带形式 Track type | 装配式单履齿（每边39块） Assembled single-grouser shoes(39/each side) |
| 履带板宽度(标准) Width of track shoes（Standard） | 500mm |
| 链轨节距 Pitch | 203mm |
| 工作装置液压系统 Implement hydraulic system |  |
| 最高工作油压 Max pressure | 12Mpa |
| 工作油泵形式 Pump type | 双联齿轮泵 Double gear pump |
| 流量 Discharge | 190升/分钟 194L/min |
| 操作阀形式 Control valve type | 滑阀式 Sliding valve |
| 工作油缸内径 X 数量 Bore of the working cylinder X number | 160mm x 1 |
| 倾斜缸内径 X 数量 Bore X NO. (Tilting cylinder) | 60mm x 1 |
| 防倾翻保护配空调司机棚 ROPS with air conditioner cab |  |
| 司机棚为六边形薄壁箱形结构。六面均装有玻璃，视野开阔，后面是推拉玻璃。空调为冷热两种功能。翻车防护架结构刚性强，推土机在非常状态时能有效的保护司机的安全。 | The cab is hexagonal thin wall box-type structure. Glass is mounted at six sides with wide view area. Pulling and pushing glass is installed at back side. Air conditioner adapts a type of cooling and heating. The structure of Rop is rigid. It can protect the driver efficiently when the bulldozer is in special condition. |