Jichai 6000 SERIES DIESEL GEN-SET                             2000kw /1500rpm

ENGINE SPECIFICATIONS

V-16,4-Stroke-Cycle-Diesel  (6000 series)

Bore--------------------------------------------------190mm

Stroke------------------------------------------------215mm

Displacement--------------------------------------97.53L

Aspiration----------------Turbocharged-After cooled

Speed Control-----------Electronic Speed governing

Rated Speed------------------------------------1500rpm

Rated power-------------------------------------2000kW

2000GF diesel generating set are powered by model H16V190ZLD-1 diesel engine produced by our company. Complying with National Standard “GB2820 General Specification for Industrial Frequency Diesel Generating Sets（eqv ISO 8528）” and with reference to “IEC34-I” rotary generator standard, the sets are equipped with high quality alternator of both home & abroad and self-made control panel according to customers requirement.

H16V190ZLD-1 diesel engine is adopted advanced governor of American WOODWARD Company. The power performance, fuel economy and reliability of the diesel engine reach world advanced level by adopting world advanced electronic control technology and lean-burnt strategy.

This genset have the features of accurate voltage regulating, good dynamic performance, less voltage waveform distortion, high efficiency, compact structure, easy to maintain, low speed, reliable and long life operation and cost-effective.

The genset can either transfer electricity directly to load or be combined in a power station with two or more than two sets. They can be used as normal and stand-by power in oil field, factory, resource development base, etc

Main Features

| Engine Design - Designed by AVL, Austria - Intellectual Property belong to Jichai - Triaxiality Finite Element Analysis - Longer Stroke, More displacement - Low Speed (1500RPM), More Reliable - Advanced World Standard Performance Generator - Brushless Excitation -AVR - Better Insulation and Protection Safety - Low Oil Pressure Auto Stop Protection - Over Speed Auto Stop Protection - Explosion Proof - High Coolant Temperature Alarm - Low Coolant Level Alarm - High Oil Temperature Alarm - Complete Electrical Protection | - Emergency Stop Device Testing - Strict Inspection and Factory Test for Each Gen-set - Start Performance Test - Load Test - Synchronization Test - Safety System Test The Whole Equipment Supply - Standard Scope of Supply - Optional Equipment List - Repairing Tools - Spare Parts - Meet the Requirement for All Accessories of Project . Technical Support & Services - Free Technical Support and Training Service - 24/7 Technical Service Tel Number - Rapid Spare Parts Supply - After Sales Service for Generator Set Lifetime |
| --- | --- |

Property Parameter Table

| Model | 2000GF |
| --- | --- |
| Engine type | H16V190ZLD-1 |
| Alternator type | 1FC series(SIEMENS technology) |
| Rated Power kW | 2000 |
| Rated Speed rpm | 1500 |
| Rated Frequency Hz | 50 |
| Rated Voltage V | 10500 |
| Rated PF | 0.8 |
| Cooling Method | Forced Water Cooled, With Radiator and Cooling Fan |
| Weight kg | 21000 |
| Dimension mm | 7150×2180×2421 |

*Rating and fuel consumption based on ISO Standard conditions (Ambient temperature at 25 degree Centigrade, Atmospheric pressure at 100kPa and Relative humidity at 30%). Site rating and fuel consumption should amend according to relative standard (The table at right side for reference )

Main Specification of Engine

| Model of engine | H16V190ZLD-1 |
| --- | --- |
| Rated Power(kW) | 2200 |
| Rated Speed(r/min) | 1500 |
| No. and layout of cylinders | 16-cylinder，60°Vee |
| Type | Four stroke, water cooled, turbocharged and after-cooled, reliable diesel engine. |
| Specific oil consumption (g/kW.h) | ≤200 |
| Specific oil consumption (g/kW.h) | ≤0.6 |
| Bore(mm) | 190 |
| Stroke (mm) | 215 |
| Total displacement (L) | 97.53 |
| Governor | Electronic Governor |
| Fuel Injection system | High pressure direct injection system |
| Starting mode | 24V DC electric starting motor |
| Direction of rotation | Counter-clockwise(facing to flywheel) |
| Lubrication method | Pressure and splash lubrication |
| Manufacturer | CNPC Jichai Co. Ltd |

Main Specification of Alternator

| Model of alternator | 1FC series(SIEMENS technology) |
| --- | --- |
| Rated power(kW) | 2000 |
| Rated voltage(V) | 10500 |
| Rated speed | 1500 r/min |
| Rated frequency(Hz) | 50 |
| Type | 6 poles, drip proof, form wound type windings, 125°C Temperature rise. |
| Exciting method | Brushless |
| Insulation class | H |
| Class of protection | IP23 |
| Cooling | A fan at the drive end draws the cooling air |
| Coupling | Elastic |
| Voltage regulator | 3 phase sensing with volts/Hz. |
| Voltage regulation | Less than +/-0.5% in steady state and less than +/- 0.5% with 3% speed change. |

Standard Scope Of Supply

Cylinder block and Cylinder head assembly

-Cylinder block assembly

-Oil sump assembly

-Cylinder head assembly

Crankshaft connecting rod system

-Crankshaft assembly

- Piston and connecting rod assembly

-Bumper assembly

-Coupling assembly

Inlet and Exhaust system

-Air filter

-Inlet System

-Exhaust System

-Valve mechanism

-Exhaust gas turbocharger

-Exhaust manifold

-Corrugated pipe

-Exhaust pipe

-Exhaust elbow

-Exhaust silencer

Fuel system

-feed pump

-Fuel cooler

-Fuel injection Pump

-Fuel injector

-Improve pressure of oil pipe

Lubricating System

-Main oil pump

-Pre-oil pump

-Oil filter

-Oil cooler

-Oil Centrifugal filter

-Pressure regulating valve and oil pipes.

Cooling System

-Cooling fan and Driven device

- Radiator

Starting System

- Starting motor

- Electric-Hydraulic Governor

Monitoring and security systems

-Monitor

-Security system

-Run/Auto/Stop control .

Alternator

-6 pole, drip proof, form wound type windings

Spares

-Normal repair tools

-Special spare parts

-Technical document

- Certificate of quality

Others

-Common base

-Battery charger

-Oversize batteries

Optional Equipment

| Fuel System -Fuel tank for 12 hours -Fuel tank for 24 hours -Fuel tank for 48 hours Cooling System -Horizontal Cooling Radiators and Cooling Fan -Pre-heating device for high and low temperature coolant -Water softening device Alternator -Other brand alternator | Spare Parts and Tools -Spare Parts for 2 years use -Repairing Tools -Lub oil for commissioning Others -Change control cable length according to site demand -Steel generator container -Installation Engineering Construction Service -Maintenance service |
| --- | --- |

Certification                                                                                     The products of CNPC Jichai have passed international certificates include European Union,  RS etc.