Preliminary Technical Proposal

for

3,300t/d Clinker New Dry Cement Production Line

China National Electric Wire&Cable Imp./Exp. Corp.

Nanjing CHOPE M&E Engineering Co., Ltd.

Content

Chapter 1:  Introduction

1.1	General Description

1.2	Technical Data and Product Standard

Chapter 2:  Production Method and Proportioning Calculation

2.1	Production Capacity

2.2	Production Method

2.3	Proportioning Calculation

Chapter 3:  Technical Scheme

3.1	Cement Making Process

3.2	Work shift of departments

3.3	List of storage facility of whole plant

3.4  List of main equipment

3.5	Power supply and distribution

3.6	Process control system

3.7  Water Supply

Chapter 4:  Main Technical and Economic Indices

Attached    Flow Sheet

.

## Introduction

  - General Description

This Technical Proposal covers a complete cement production line from raw material crushing to cement packing and delivering.

  - Technical Data and Product Standard

    - Production Capacity

Clinker output: 3,300t/d

    - Product Standard

Product complies with China National Standard GB175-1999 (equal to British Standard BS12-1989 or American Standard ASTM150-92).

    - Heat Consumption

Clinker burning: 3,094 kJ/kg-cl（740 kcal/kg-cl）

    - Electric Power Consumption

Finished cement product: 98 kWh/t

    - Exhaust Gas Handling

In an environment-friendly design, electrostatic precipitators are used for handling the flue gas discharged from preheater system and grate cooler and impulse bag filters are used at where necessary and applicable. The density of dust of the flue gas coming out from the electrostatic precipitators as well as that from bag filters is less than 50mg/Nm3.

    - Plant site conditions

(1) Temperature

Yearly Average Temperature 			: 22 ℃

Maximum Temperature					: 35 ℃

Minimum Temperature 			        : 3 ℃

(2)	Humidity

Yearly Average Humidity				: 64%

Average of the hottest month			: 76%

Average of the coldest month			: 51%

(3)	Rain-Fall

Average rain-fall (Yearly):				132.3 mm

Rain season monthly average: 			134.2 mm

Dry season monthly average: 			34.5 mm

(4)	Wind

The way of strongest winds: 			NW

Max. wind velocities at site: 				22.65m/s

(5)	Snow Load:								Rarely snows

(6)	EARTHQUAKE ZONE:					1st degree quake zone,

    - Water supply

PURCHASER will supply the water for production. Water source available is groundwater.

    - Fuel

The bituminous coal will be used as burning fuel.

    - Power supply

PURCHASER will supply the Main Power Voltage of 34kV for this project and main substation.

  - Others

※	Language

English will be used in all documentation.

※	Measuring units

All dimensions and other numerical data will be in the metric system.

※	Applicable codes and standards

All design documents will be in conformity with Chinese code and standards. All equipment and materials provided by the SUPPLIER will be in conformity with manufacturer’s standards.

## Production Method and Proportioning Calculation

  - Production Capacity

Clinker output:	 3,300 t/d

1,000,000 t/y

Portland Cement output: 1,200,000 t/y

  - Production Method

The dry process production line with 4.4×68m rotary kiln equipped with precalciner system and 5-stage cyclone preheater is used for the project.

  - Proportioning Calculation

    - Chemical Composition of Raw Materials and Fuel (%)

Raw Materials chemical Composition

| % | Limestone | Clay | Sand | Iron ore | Fly Ash of coal |
| --- | --- | --- | --- | --- | --- |
| L.O.I. | 42.03 | 13.58 | 4.60 | 4.67 |  |
| SiO2 | 3.13 | 57.14 | 81.20 | 15.24 | 53.46 |
| Al2O3 | 0.92 | 12.61 | 6.80 | 12.15 | 30.23 |
| Fe2O3 | 0.46 | 4.66 | 3.20 | 63.10 | 5.99 |
| CaO | 52.78 | 8.01 | 1.60 | 2.60 | 8.13 |
| MgO | 0.43 | 2.40 | 0.60 | 1.20 | .11 |
| K2O | 0.04 | 0.40 | 1.10 | 0.10 | .26 |
| Na2O | 0.09 | 0.34 | 0.30 | 0.19 | .6 |
| SO3 | 0.12 | 0.66 | 0.100 | 0.75 | 1.26 |
| Chlorides Cl- | 0.001 |  | 0.010 |  |  |
| Total | 100.00 | 99.74 | 99.51 | 100 |  |

Presumed Approximate Analysis of the proposed Coal

| Ma,r | Mad | Aad | Vad | FCad | Qnet,ad | St,ad |
| --- | --- | --- | --- | --- | --- | --- |
| 12% | 1.0% | 18% | 25% | 56% | ≥5800 kcal/kg | ≤1.5% |

Note: Chemical composition of Raw Materials and Fue is presumed.

    - Determination of Clinker Ratio

The range of clinker ratio is as follows:

LSF=93±1, 	SM=2.5±0.10, 	IM=1.60±0.1.

The heat consumption for clinker burning is 3094kJ/kg (740kCal/kg).

    - Proportioning Scheme

The raw materials proportioning ratio, chemical composition of raw meal and clinker, clinker ratio and mineral composition are as follows:

|  | composition |  |  | Proportioning （%） |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Raw Mix | Clinker |  | dry | wet |  |
| L.O.I （%） | 36.02 |  | Limestone | 80.00 |  |  |
| SiO2 （%） | 13.81 | 22.31 | Clay | 16.40 |  |  |
| Al2O3 （%） | 3.13 | 5.47 | Iron ore | 1.50 |  |  |
| Fe2O3 （%） | 2.15 | 3.41 | Sand | 2.10 |  |  |
| CaO （%） | 43.61 | 66.78 |  |  |  |  |
| MgO （%） | 0.77 | 1.18 |  |  |  |  |
| K2O （%） | 0.12 | 0.19 |  |  |  |  |
| Na2O （%） | 0.13 | 0.21 |  |  |  |  |
| SO3 （%） | 0.21 | 0.35 |  |  |  |  |
| Cl- （%） | 0.001 | 0.002 |  |  |  |  |
| Total (%) | 99.94 | 99.91 | Total | 100.00 |  |  |
| IM | 1.46 | 1.60 |  | Coal |  |  |
| SM | 2.62 | 2.50 | Fuel (%) | 0.00 |  |  |
| S/A | 4.41 | 4.08 | Net value of fuel | 24253.3 | kJ/kg |  |
| KH | 0.971 | 0.901 | Burning consumption | 3094.4 | kJ/kg cl. |  |
| LSF |  | 93.9 | Fuel consumption | 131.5 | kg fuel/t clinker |  |
| HM |  | 2.14 | Ash in clinker(%) | 2.30 | （%） |  |
| C3S (%) |  | 59.69 | R. M. consumption | 1.5270 | t R.M./t cl. |  |
| C2S (%) |  | 18.96 | Moisture of raw mix |  | (%) |  |
| C3A (%) |  | 8.73 | Remark: |  |  |  |
| C4AF (%) |  | 10.38 |  |  |  |  |
| Na2O+K2O (%) |  | 0.41 |  |  |  |  |
| 1400℃ liquid volume(%) |  | 25.24 |  |  |  |  |
| S/R | 1.43 |  |  |  |  |  |
| E-SO3(g/100Kg clinker) |  | 345.3 |  |  |  |  |

- Remarks

(1)	In this project, the limestone, clay, sand and iron ore will be used as the proposed raw materials and bituminous coal as burning fuel. Above raw materials and coal may satisfy the requirement of new dry cement clinker production line with 5-stags preheater and precalciner.

(2)	The clinker modules and noxious components shall satisfy the following conditions for the normal operation of the kiln system:

Modules in the clinker:

KH:      0.89~ 0.91

SM:      2.4  ~ 2.6

IM:      1.5  ~ 1.7

Noxious components in the raw meal:

K2O+Na2O		 < 1%

Cl-		   		 < 0.015%

Ratio of sulphur and alkali in the clinker:

MolSO3

SG = 					  ≤ 1

MolK2O+1/2MolNa2O

(SO3 included from the raw meal and fuel)

If SG>1, the excess SO3 in the calculated clinker shall be less than 400 g/100kg clinker.

(3)	Requirements of the raw materials and fuel

Usually, the proposed raw materials and fuel must conform to the following requirements:

Raw materials

|  | Limestone | Clay | Iron ore | Sand |
| --- | --- | --- | --- | --- |
| CaO (%) | > 48 |  |  |  |
| MgO (%) | < 3 | < 3 |  |  |
| K2O+Na2O (%) | < 0.6 | < 4.0 | < 4 | < 2 |
| SO3 (%) | < 0.5 | < 1 | < 2 | < 1 |
| Cl- (%) | < 0.015 |  |  |  |
| SM |  | 2.5 ~ 3.5 |  | >4 |
| IM |  | 1.5 ~ 3.5 |  |  |
| Fe2O3 (%) |  |  | > 45 |  |
| Al2O3 (%) |  |  |  |  |
| Moisture (%) | < 3 | < 15 | <12 | < 8 |
| Size (mm) | ≤ 1200 | ≤ 350 | ≤400 | ≤ 25 |

(4)	The grindability (Bond Index) of the raw mix should be less than 10 kW•h/t.

(5)	Burning Fuel      coal

Net heat value as air dried	> 5800kcal/kg

Volatile as air dried			> 25%

Moisture as pulverized coal:	< 1%

Total sulfur as air dried:		< 1.5%

(6)	The formulas of common clinker modules are the following:

Lime saturation factor (KH):

Silica ratio (SM):

Alumina ratio (IM):

    - Material Balance

| Material | Dry ratio | Moisture | Consumed amount for clinker (t/t) |  | Material Balance (t) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Dry |  |  | Wet |  |  |
|  | % | % | Dry | Wet | Hour | Day | Year | Hour | Day | Year |
| limestone | 80.0 | 2 | 1233.97 | 1259.16 | 169.67 | 4072.11 | 1262353 | 173.13 | 4155.21 | 1288116 |
| Clay | 16.4 | 12 | 252.96 | 287.46 | 34.78 | 834.78 | 258782 | 39.53 | 948.62 | 294071 |
| Iron ore | 1.50 | 10 | 23.14 | 25.71 | 3.18 | 76.35 | 23669 | 3.53 | 84.84 | 26299 |
| Sand | 2.10 | 8 | 32.39 | 35.21 | 4.45 | 106.89 | 33137 | 4.84 | 116.19 | 36018 |
| Raw meal |  |  | 1542.47 |  | 212.09 | 5090.13 | 1577942 |  |  |  |
| Clinker |  |  |  |  | 137.50 | 3300.00 | 1023000 |  |  |  |
| Burning coal |  | 10 | 131.53 | 146.15 | 18.09 | 434.06 | 134557 | 20.10 | 482.28 | 149508 |
| Gypsum |  | 3 |  |  | 9.92 | 238.10 | 73810 | 10.23 | 245.46 | 76092 |
| Slag |  | 6 |  |  | 9.06 | 217.36 | 67381 | 9.63 | 231.23 | 71682 |
| Trass |  | 3 |  |  | 17.35 | 416.45 | 129100 | 17.89 | 429.33 | 133092 |
| Clinker |  |  |  |  | 137.50 | 3300.00 | 1023000 |  |  |  |
| Cement(A) |  |  |  |  | 73.10 | 1754.39 | 543860 |  |  |  |
| Cement(B) |  |  |  |  | 99.21 | 2380.95 | 738095 |  |  |  |
| Cement (total) |  |  |  |  | 172.31 | 4135.34 | 1281955 |  |  |  |

Remarks:

(1) Kiln running rate: 310d/a;

(2) Material consumption: 1.527kg raw meal/kg clinker;

(3) Net heat value of Coal as air dried: 24253kJ/kg;

(4) Heat consumption: 740kcal/kg clinker (3094kJ/kg clinker);

(5) Burning fuel is bituminous coal.

(6) Cement proportioning

|  | Cement（A） | Cement（B） |
| --- | --- | --- |
| Clinker (%) | 95 | 70 |
| Gypsum (%) | 5 | 5 |
| Slag (%) |  | 10.00 |
| Trass (%) |  | 15.00 |

Cement (A) is Portland cement and cement (B) is composite cement.

- Technical Scheme

  - Cement Making Process

This project aims to build a 3,300t/d cement production line from raw materials crushing to cement packing and delivery. The proposal is made for a 3,300t/d new dry process production lines with suspension cyclone preheater. Most of the major equipment used in the production line is manufactured in China in recent years based on the licenses from the Western countries, e.g. Some of key equipment will, if necessary, be supplied by the Western countries,

    - Limestone and clay Crushing

In this scheme, a hammer crusher will be installed in the plant site. Limestone  from mine is transported via dumper trucks to crusher feeding hopper, under which a PB 2000 apron feeder is installed and one mobile loader is used for feeding the limestone temporarily stored in nearby area. Limestone is fed via apron feeder to hammer crusher for crushing. The crusher has a capacity of 800t/h. The apron feeder is driven by frequency-variable motor and its speed is monitored and controlled through instruments in the control cabinet, so that the crusher will work efficiently in nearly full load. Crushed limestone is transported via B1400 belt conveyor to a B1000 belt conveyor ,mix with crushed clay and then to the limestone & clay preblending. an electronic belt weigher is used for measuring the mixture(limestone & clay).

Clay is transported via dumper trucks to crusher feeding hopper, under which a PB 1000 apron feeder is installed and one mobile loader is used for feeding the clay stored in shed nearby . Clay is fed via apron feeder to impact crusher for crushing. The crusher has a capacity of 180t/h. The apron feeder is driven by frequency-variable motor. Crushed clay is transported via B800 belt conveyor to a B1000 belt conveyor, mix with crushed limestone and then to the limestone & clay preblending. an electronic belt weigher is used for measuring the clay.

    - Coal & Iron ore crushing and sand conveyor

Coal & Iron ore are charged to the receiving hopper via dumper trucks or loader and then fed into the crusher by apron feeder with variable speed respectively, the additive materials are crushed by hammer crusher into proper size. The input size of the materials shall be less than 400 mm and the output is less than 35 mm, the capacity of the crusher is 150t/h. Sand are charged to feeding hopper via loader under which a bar gate is installed . Crushed materials & sand are conveyed to the Additive Materials preblending.

    - Limestone & Clay Preblending

A 325 longitudinal limestone & clay preblending yard will be built in a storing capacity of 2×38000 tons of mixture and a stockpile of 9800 tons of pure limestone. Mixture & pure limestone are stored in preblending stockpile via boom stacker. A bridge scraper reclaimer is employed. There are two stockpiles, one for stacking and the other for reclaiming. The capacity of the stacker is 800t/h and that of reclaimer is 500t/h. Preblended mixture are taken by the reclaimer and then conveyed to proportioning station via belt conveyor. Pure limestone is fed into feeding hopper via loader then conveyed to proportioning station by belt conveyor.

    - Coal & Iron ore Preblending

A longitudinal preblending yard with storing capacity of 7000 tons for sand, 6000t for iron ore, 2×10000t for coal will be built. A side reclaimer is employed for reclaiming all additive materials and coal. The capacity of the stacker is 200t/h and that of reclaimer is 100t/h. The iron ore and sand will be conveyed to raw materials proportioning station by belt conveyor. The coal will be conveyed to coal bin of coal mill via belt conveyor.

    - Raw Materials Proportioning

Four proportioning bins are planned in the raw materials proportioning station to store limestone & clay, pure limestone ,iron ore and sand (limestone & clay 950t, pure limestone 600t, iron ore 750t, sand 600t). The materials are fed via the weighing feeder at the bin bottom according to the proportion ratio respectively. The proportioned raw materials are conveyed via belt conveyor to raw mill. The chemical composition of the raw materials mix will be monitored by XRF analysis on samples taken at the vertical mill cyclone outlet on. On the basis of analysis results, raw meal composition will be controlled by acting on the weigh feeders to assure the proper blend of materials.

    - Raw Material Grinding and Transporting

A vertical mill with a high efficiency separator will be used for raw materials grinding and drying. The capacity of the raw mill is ~260t/h, which includes reserve capacity for worn rollers/table liners. The main motor power is approx. 2800kW. The raw meal fineness is 12% R90.

A metal separator is installed before the raw materials are fed to the vertical mill to eliminate ferro-metallic parts. A metal detector is placed 5m from the end of the feeding belt conveyor. In case of an emergency, the raw materials with metallic parts are discharged through a two-way gate into a scrap bin.

The ground fine particle will be carried by air to the cyclone separator and the coarse particle will be collected by the integrated separator and returned to vertical mill for regrinding. The dust-laden air out of the cyclone separator will be treated in an EP before being emitted into the atmosphere. The dust concentration in the dedusted gas will be lower than 50mg/Nm3 on design. The raw meal product captured in the cyclone separator will be sent to the homogenizing silo. Dust collected by the air conditioning tower & EP will always be transported to the silo.

Water spray system in the vertical mill will also be necessary for stability of the mill and in order to control the mill outlet temperature. A raw mill fan is located between the cyclone separator and the EP. The circuit will be designed to allow gas re-circulation from the raw mill fan back to the inlet of the mill. The duct of the mill inlet is to be equipped with a fresh air intake in order to adjust the mill inlet temperature.

Preheater exhaust gases will be used to dry the raw materials in the vertical mill. No auxiliary air heater is requested. When the vertical mill is in operation, the preheater exhaust gases, after being lowered to a proper temperature by the conditioning tower, will be mixed with that from the vertical mill and then sent to the EP. Totality of preheater exhaust gases can be allowed to the EP when the raw mill is shut down without interfering with kiln operations. The EP fan is located downstream of the EP and the dedusted gases will be emitted into the atmosphere through a stack.

All the main equipment is outdoor. Proper access for maintenance trucks and cranes entering into the area surrounding the raw mill, dust collecting system, homogenizing silo, preheater and fans will be considered in the layout.

    - Raw Meal Homogenizing Silo

A 18m homogenizing silo of 10500t capacity will be installed for blending and storing the raw meal before it is fed to the kiln system. In order to guarantee the silo blending effect, the storing of the silo must be remained up to 5000t raw meal during nominal kiln operation.

The raw meal product captured in the cyclone separator will be transported into the silo by air slides and elevator . Extraction from the bottom of the silo shall be via discharge air slides and dosing valves to the kiln feed bin. The fluidization/extraction air is supplied from roots blowers. The silo discharge system must be adequately controllable in order to keep the kiln feed bin at a constant weight level. A manhole in the silo allows checking the air slides of the silo bottom.

The kiln feed bin shall be mounted on load cells, and a gravimetric metering system controls the flow of raw meal into the preheater. For an emergency stop of preheater feed and for start-up procedures, the preheater feeding elevator is equipped with a by-pass flap, which sends the raw meal back to the homogenizing silo.

    - Burning -system

Burning system will consist of a double series 5-stage cyclone preheater with precalciner, a 4.4×68m rotary kiln, as well as a grate cooler. The system capacity will be designed for 3300t of clinker per day and the heat consumption will be 740kcal/kg.cl on the basis of 400 kcal/kg.cl of reaction heat of clinker formation. The cooler will be designed for the outlet clinker temperature of 65℃ plus the ambient temperature.

The preheater with precalciner is the technique patent and product of C-HOPE. Material feeding ducts between the stages are equipped with pendulum flaps and splash boxes. The cyclones will be sized so that the pressure drop of the preheater system will be less than 5500Pa and the exhaust gas temperature out of the preheater will be 330~350C in nominal operation. The weighed raw meal will be fed into preheater for preheating and partial decarbonation and then into precalciner for calcination. The decarbonation rate of CaCO3 in the raw meal at the kiln inlet is over 90%. The raw meal is further decarbonated and burned in the kiln.

Steel stairs and platforms with adequate headroom and space for maintenance of all equipment and manholes, a service elevator for man and bricks and spare parts if necessary, etc will be taken into account during the design of the steel structure of the preheater tower.

The high temperature fan will be equipped with a variable speed drive. The location of air blasters is to be optimized during continuous running. Gas analyzers with independent cooling systems are located in the duct after the precalciner.

The rotary kiln shall be fired with bituminous coal. The drive system will be mechanical. The tertiary air duct, installed along the side of the kiln, takes the hot air preferably from the kiln hood to the precalciner. Free access for maintenance trucks and cranes entering into the area surrounding the rotary kiln will be considered in the layout.

The cooler has three grates, the crusher which has the same width as the cooler grates will be of hammer type and installed at the end of the grates. Water spray system within the cooler will also be necessary for dedusting and in order to control the EP inlet temperature. Electrical equipment, a central lubrication group, etc of the cooler will be included. The grate cooler and chain conveyor below the cooler chamber will be installed above ground level. Cooler air fans are to be equipped with silencers and adjust valves.

At the kiln outlet, the clinker with temperature over 1350C will enter the grate cooler for quenching. Clinker lumps are crushed by the hammer crusher at the end of the grates. Recuperated hot air is utilized as secondary air in the kiln and tertiary air for the precalciner. The part is sent to the coal mill for coal drying and the rest exhauste gas from the grate cooler shall be cleaned in a EP which will be designed for the dust concentration in the dedusted gas less than 50mg/Nm3. Except the water spray system within the cooler, a fresh air intake at the duct of the EP inlet will be designed in order to cool down gases from cooler to a temperature acceptable for the EP. The EP fan is located downstream of the EP and the dedusted gases will be emitted into the atmosphere through a stack. The dust collected by the EP shall be fed back to the bucket chain conveyor under the clinker crusher of the cooler.

Stairs and platforms with adequate space for maintenance of the cooler and the EP, etc will be taken into account during the design of this department.

    - Clinker storage and transporting

A 40m clinker silo of 45000t capacity will be used to store the clinker. The clinker silo has three extraction galleries with low speed belt conveyor above ground level.

The clinker out of the cooler will be transported by the bucket chain conveyor to the silo. The bag filter on the top of the silo is sized for the feeding by the bucket chain conveyor and thermal air expansion in the storage. The clinker extracted from the silo will be transported by belt conveyors to the cement proportioning. Necessary dedusting filters will be designed at the chutes of clinker on to belt conveyors to the cement proportioning.

    - Gypsum crushing & slag, trass conveying

Gypsum is unloaded into feeding hopper by dumper or mobile loader, A jaw crusher will be equipped for crushing gypsum. The crusher has a capacity of 30~50t/h. Slag, trass are storied a longitudinal shed. These materials will be sent to the cement proportioning by belt conveyer and bucket elevator.

    - Cement Proportioning Station

Four proportioning bins are planned in the cement proportioning station to store clinker, gypsum and slag, trass.(clinker 1200t, gypsum 1000t, slag 700t, trass 1000t).The materials are fed via the weighing feeder at the bin bottom according to the proportion ratio respectively. Bins and extraction must be dedusted. The proportioned materials are conveyed via belt conveyor to cement grinding.

    - Cement Grinding

Two cement grinding system will be adopted and each system includes a roller press and a 4.212.0m cement mill in closed circuits with O-Sepa high efficiency classifier. The nominal capacity of each cement mill system will be 120t/h of Ordinary Portland Cement at fineness of 320m2/kg. The main motor power of each mill is about 2800kW.

Proportioned mixture are conveyed via belt conveyor and elevator below the proportioning bins to feeding bin, which is equipped with three load cells, above the roller press. The weight signal sent by the cells serves as the feedback signal of materials flow to control the feed volume of belt weigher under the proportioning bins for mixture. Materials from the roller press are sent to V-shaped powder separator. Coarse particles return to the feeding bin above the roller press. Fine product  taken by the airflow is sent to cyclone collector and then to the ball mill for grinding. Those cannot be caught by cyclone collector are sent to O-SEPA separator of the ball mill by airflow.

Materials ground by ball mill are transported to N2500 O-SEPA separator by high efficiency bucket elevator. The coarse particles separated return to ball mill for regrinding while the fine product is collected by the high efficiency bag filter behind the separator and then conveyed to cement silo via an air slide and bucket elevator.

    - Cement storage and bulk loading

8-15m cement silos will be used to store the cement, and the total storage capacity is 40000t. These silos will be installed with dedusting system, necessary accessories, fluidization and extraction systems.

    - Cement packing & Bulk cement loading

The cement packing system will include three packers of 8-spout, the 90t/h capacity each, and can pack the daily cement product completely in 12~15 hours. Each system will include vibrating screen, buffer bin and weighing, etc. There are 3 bulk loading system of 100t/h design capacity for truck.Each bulk cement bin has the capacity 1600 tons.

The cement from extraction systems under the cement silo will be delivered to the buffer bin by air slide and the bucket elevator and vibrating screen, before being fed into each packer. The bagged cement can be loaded directly or stored in storehouse temporarily which has the capacity 2000 tons.

    - Pulverized coal preparation

A vertical mill with a high efficiency separator will be used for raw coal grinding and drying. The nominal capacity of the mill is 25t/h of coal pulverized at 10% reject on 80 microns sieve.

The raw coal bin will be equipped with a load cell. Bin and extraction must be dedusted. The raw coal will be adjusted by weigh feeders. A metal separator is installed before the raw coal is fed to the vertical mill to eliminate metallic parts. The dust collected by the bag filter will be the qualified coal powder, which will be transported either to a bin for kiln or to a bin for precalciner. The dedusted air from the bag filter will be to the chimney. A fresh air inlet with damper will also be included in the mill air circuit. The design dust concentration in the dedusted gas will be lower than 50mg/Nm3.

The cooler exhaust gases will be used to dry the raw coal in the vertical mill. An auxiliary air heater is requested.

The pulverized coal extracted from the bins by two sets of weighing and feeding systems will be pneumatically transported to the kiln and precalciner burners. The capacities will be 7~9t/h for the kiln burner and 10~12t/h for the precalciner burner. The transporting air is supplied from Roots blowers.

The workshop will also include necessary explosion valves, CO and temperature detection and CO fire protection, etc.

    - Utilities

The project will also include necessary utilities sections namely:

Two truckscales of 90t located at plant entrance

Three truckscales of 90t located under bulk cement bins

Air compressor station , A center labouratory and a small pilot plant,etc.

3.2		Work shift of departments

| No. | Department | Day (day/week) | Work hour per daily balance (hr/day) |
| --- | --- | --- | --- |
| 1 | Limestone crushing | 6 | 6.2 |
| 2 | Clay crushing | 6 | 6.2 |
| 3 | Limestone & clay Preblending Incl.: Stacker Reclaimer | 6 7 | 6.2 9.7 |
| 4 | Coal, iron ore crushing | 6 | 4.5 |
| 5 | Coal, iron ore and sand Preblending Incl.: Stacker Reclaimer | 6 7 | 5.0 6.9 |
| 6 | Raw materials proportioning station Incl.: Top of bins Bottom of bins | 7 7 | 12.5 20.4 |
| 7 | Raw materials grinding | 7 | 20.4 |
| 8 | Exhaust gas treatment | 7 | 24 |
| 9 | Raw meal homogenizing silo Incl.: Top of silo Bottom of silo | 7 7 | 20.4 24 |
| 10 | Burning system | 7 | 24 |
| 11 | Clinker storage Incl.: Top of silo Bottom of silo | 7 7 | 24 6.6 |
| 12 | Gypsum crushing | 7 | 6.2 |
| 13 | Cement grinding | 7 | 17.3 |
| 14 | Cement storage Incl.: Top of silo Bottom of silo | 7 7 | 17.3 Based on sales |
| 15 | Cement bulk loading | Based on sales | Based on sales |
| 16 | Cement packing and bag loading | Based on sales | Based on sales |
| 17 | Pulverized coal preparation | 7 | 19.3 |
| 18 | Coal meal weighing | 7 | 24 |
| 19 | Central control room & lab. | 7 | 24 |
| 20 | Air compressor station | 7 | 24 |

  - List of storage facility of whole plant

| No. | Name | Storage type | Specification (meter) | Capacity (ton) | Duration (day) |
| --- | --- | --- | --- | --- | --- |
| 1 | Limestone & clay | Preblending pile | 325×54 | 2×38000 | 2×7.8 |
|  |  | Proportioning bin | 10 | 950 | 4.7h |
| 2 | Pure limestone | preblending pile |  | 9800 | 23 |
|  |  | Proportioning bin | 8 | 600 | 1.5 |
| 3 | Iron ore | preblending pile | 253×46.5 | 6000 | 70 |
|  |  | Proportioning bin | 8 | 750 | 8.5 |
| 4 | sand | preblending pile | 253×46.5 | 7000 | 58 |
|  |  | Proportioning bin | 8 | 600 | 5.0 |
| 5 | Coal | Preblending pile | 253×46.5 | 2×10000 | 2×20 |
|  |  | Raw coal bin |  | 80 | 4h |
| 6 | Raw meal | Homogenizing silo | 18 | 10500 | 2.1 |
| 7 | Clinker | Silo | 40 | 45000 | 13.6 |
|  |  | Proportioning bin | 10 | 1200 | 8.7h |
| 8 | Gypsum | stockpile | 48×40 | 6600 | 25 |
|  |  | Proportioning bin | 10 | 1000 | 4 |
| 9 | Slag | Shed | 54×60 | 6800 | 28 |
|  |  | Proportioning bin | 10 | 700 | 3 |
| 10 | Trass | Shed | 54×72 | 8000 | 18.6 |
|  |  | Proportioning bin | 10 | 1000 | 2.3 |
| 11 | Cement | Silo | 8-15 | 8×5000 | 9.5 |
| 12 | Cement | Bulk bin | 3-6x6 | 3-1600 | 9h |

  - List of main equipment

| No. | Name | Specification and Size | Q'ty | Capacity (t/h) |
| --- | --- | --- | --- | --- |
| 1 | Limestone Crusher | Single stage, hammer type Feed size:<100010001000mm Product size: <80mm R90% Main motor: 630kW | 1 | 800 |
| 2 | Clay crusher | Impact type | 1 | 180 |
| 3 | Limestone Stacker | Boom type side stacking | 1 | 800 |
| 4 | Limestone reclaimer | Bridge scraper | 1 | 500 |
| 5 | Coal & iron ore Crusher | Impact type | 1 | 150 |
| 6 | Coal & iron ore Stacker | Boom type side stacking | 1 | 200 |
| 7 | Coal & iron ore reclaimer | Side scraper | 1 | 100 |
| 8 | Raw mill | Vertical type Feed size: <75mm Product fineness: <12% R90 Feed moisture: max.6.0% Product moisture: <1.0% Main motor: ~2800kW | 1 | 260 |
| 9 | Raw mill fan | Air flow: ~550000m3/h Pressure: 11000Pa Air temperature: 100C, max.150C Dust inlet: <100g/Nm3 Main motor: ~2100kW | 1 |  |
| 10 | Preheater fan | Air flow: 580000m3/h Pressure: 8000Pa Air temperature: 310C, max.450C Dust inlet: <80g/Nm3 Main motor: ~2200kW | 1 |  |
| 11 | EP for kiln and raw mill | Air flow: ~580000m3/h Inlet air temp.: 100~150℃ Dust inlet: <80g/Nm3 Dust outlet: <50mg/Nm3 | 1 |  |

List of main equipment in whole plant (continued)

| No. | Name | Specification and Size | Q'ty | Capacity (t/h) |
| --- | --- | --- | --- | --- |
| 12 | Preheater and precalciner | Double series, 5-stage C1: 2-5450mm C2, C3: 2-5450mm C4, C5: 2-5700mm Precalciner: 5900mm Pressure loss: <5500Pa Outlet air temp.: <330~350C | 1set | 3300t/d |
| 13 | Rotary kiln | 4.4×68m Slope: 4% Main motor: 420kW | 1 | 3300t/d |
| 14 | Grate cooler | Effective grate areas: ~83.16m2 Inlet clinker temp.: ~1350C Outlet clinker temp.: 65C+ ambient Outlet clinker size: <25mm | 1 |  |
| 15 | EP for cooler | Air flow: ~435000m3/h Inlet air temp.: 220~350C Dust inlet: <20g/Nm3 Dust outlet: <50mg/Nm3 | 1 |  |
| 16 | Cement mill | Roller press + 4.2×12m ball mill Feed size: <25mm Product fineness: 3200cm2/g Ball mill main motor: 2800kW | 2 | ~120 (each) |
| 17 | Classify | O-Sepa, N-2500 Capacity: 90150t/h Feed rate: max. 450t/h Air rate: 2500m3/min. Main motor: 110kW | 2 |  |

List of main equipment in whole plant (continued)

| No. | Name | Specification and Size | Q'ty | Capacity (t/h) |
| --- | --- | --- | --- | --- |
| 18 | Bag filter for classify | Air flow: 157000m3/h Dust inlet: <1300g/Nm3 Dust outlet: <50mg/Nm3 | 2 |  |
| 19 | Coal mill | Vertical type Feed size: <40mm Product fineness: <12% R90 Feed moisture: max.12% Product moisture: <1% Main motor: ~420kW | 1 | 25 |
| 20 | Bag filter for classify | Air flow: 100000m3/h Dust inlet: <720g/Nm3 Dust outlet: <50mg/Nm3 | 1 |  |
| 21 | Bulk loader for truck |  | 3 | ~100(each) |
| 22 | Cement packer | 8-spout rotary | 3 | 90(each) |

  - Power supply and distribution

    - In addition to electricity equipment directly associated with Equipment supplied, the project will also include:

        - Main substation and external feeding line are excluded

        - Medium and low voltage power distribution

        - Electrical protection system

        - Emergency power system

        - Lighting

    - Power supply

The power is supplied by public grid to the 34kV substation in the plant.

A 800kVA diesel power generator will be installed near the preheater electric room for the safety of the rotary kiln during the emergent shutdown. The emergent power will supply the power for the kiln auxiliary drive motor, lubricating system of kiln bearing cooling system necessary lighting, maintenance and fire fighting, etc.

(1)	Voltage grade

Voltage of incoming power supply is 34 kV, 3 phases, grounded Neutral, 50HZ.

M.V. power distribution  		6.3 kV

M.V. motor               	6 kV

L.V power distribution    		0.4/0.23 kV

L.V motor                	380 V

DC operating power        	DC 220V

DC motor                  	DC 220V

(2)	Estimated Load

Overall installed power:				25600kW

Thereinto: H.V. motor：			18380kW

Calculated load:						18300kW

Natural power factor：				0.7

Power factor after compensation：		0.95

Consumption power of per ton cement:	98kWh/t cement

Annual comsumption power:			1.25108kWh

Demand factor:						0.70

    - Power distribution

Unmanned running mode will be used in the general substation during operation. All the running signals, alarm signals, and data from any device will be processed by PLCs and sent via data highway to the CCR. The control and supervision of the general substation will be executed in the CCR.

There will be 6 electric rooms respectively for raw mill, preheater, kiln hood, coal mill, cement mill and cement packing.

The electric rooms will receive the power from the general substation via the 6kV cables.

The transformers for power distribution will be totally enclosed.

The 6kV medium tension switchgear will be metal enclosed drawable.

The low tension MCC drawer cabinets will be installed with the intelligent relays, executing the overload, undervoltage, lack of phase, earthing and damping protection and load current and earthing current display, etc.

    - Electric drive and control

All the signals about the equipment running status will be sent to the PLCs. The motors will be sequentially started or stopped according to the process flow as initiated by the commands from the operator's keyboard. The motors will be stopped sequentially once a fault signal is received and thus an alarm signal sent out.

The kiln and the kiln ID fan drive will be run by way of the variable speed regulation.

The mill motor and other big slip ring motors will be started by the liquid rheostats.

The changeover switchbox "CENTRAL-STOP-LOCAL" will be installed adjacent to the motors to avoid misoperation of the motors during the maintenance and repair of the equipment.

    - MT motor list

| No. | Driven Equipment | Q’ty | Rating Power (kW) |  |
| --- | --- | --- | --- | --- |
|  |  |  | Single | Sub-Total |
| 1 | Limestone crusher | 1 | 630 | 630 |
| 2 | Additive material and coal crusher | 1 | 280 | 280 |
| 3 | Raw mill | 1 | 2800 | 2800 |
| 4 | Fan for raw mill | 1 | 2100 | 2100 |
| 5 | High temperature fan | 1 | 2200 | 2200 |
| 6 | Fan after EP of pyro-system | 1 | 1000 | 1000 |
| 7 | Fan after EP of clinker cooler | 1 | 350 | 350 |
| 8 | Roller press | 2 | 500 | 1000 |
| 9 | Fan for roller press system | 2 | 250 | 500 |
| 10 | Cement mill | 2 | 2800 | 5600 |
| 11 | Fan for bag collector after cement mill | 2 | 500 | 1000 |
| 12 | Coal mill | 1 | 420 | 420 |
| 13 | Fan for bag collector after coal mill | 1 | 500 | 500 |
|  | Total | 17 |  | 18380 |

  - Process control system

In addition to automation equipment directly associated with equipment supplied, the project will also include:

        - Central control room

        - Control system

        - Communications system

    - Principle of design

In the quotation, the advanced equipment and technology for automatic control system in the world is proposed. The design principle is reliable and practical. The instrumental control is used for the auxiliary department.

    - Process control

A distributed control system will be adopted. The plant will set one central control room (CCR) and four local control stations (in the electric rooms). The supervision and operation will be carried out from the CCR. The report and alarm message and other management documents can be printed.

    - Raw meal quality control system

The samples will be taken locally by an automatic sampler from a point between the raw mill and homogenizing silo and then transported manually to the central lab, where it will be semi-automatically prepared and sent to an eight-channel X-Ray analyzer. The results analyzed will be sent to a proportioning computer. The computer will calculate the ratio of raw materials and send out the set value to constant feeder according to the chemical compositions and raw meal module required (LSF, SM, AM).

    - Motor control

The motor control will be carried out by the local control stations. Each motor could be controlled locally or in CCR, and each control circuit is attached with short circuit under voltage and overload protection.

    - Measuring and weighing

      - Gas analyzer

The burning in the precalciner and kiln will be supervised by an analyzer for CO, O2 set at the kiln inlet and an analyzer for CO, O2 set at the exit of precalciner to ensure the optimal air rate and fuel ratio, to protect the environment and to improve the burning efficiency and running performance of the kiln.

      - Industrial TV

The cameras and monitors will be used for limestone preblending, raw grinding, kiln hood, cooler and cement mill.

      - Kiln shell scanning system

A kiln shell infrared scanner will be adopted for monitoring the temperature of kiln shell, coating and refractories. Alarm will be sent out if abnormal condition happens and the brick life time could be prolonged and the kiln operation could also be optimized.

      - Weighing

All raw materials and raw coal will be weighed as delivered into the plant. Weighers will be used at necessary points in the process for production control.

      - Power consumption

The consumption of power of consumer higher than 55kW will be measured.

    - Field instrument and control parameter

The reliable, stable and proper primary detecting elements will be selected for the measuring of temperature, pressure, flow rate, material level, voltage, current, power, speed, valve position and vibration, etc.

    - Selection and laying of cable

Shielded cable as the signal cable for field instrument will be used. Cable inside room will be laid on racks or in conduits, and outside will be embedded and in trenches. Shielded cover will be paved on the signal cable to separate from the power cable if they are laid together. The control cable will be more than 300mm away from the field instrument cable, if they are laid in parallel.

    - Earthing

Separate earthing system will be used for CCR and all different local control stations.

    - The DCS is divided into 4 control areas:

Area 1	Raw materials proportioning station
Raw materials grinding

Area 2	Preheater 
Raw meal homogenizing silo
Raw meal kiln feeding
Exhaust air treatment

Area 3	Rotary kiln
Cooler
Pulverized coal preparation

Area 4	Cement grinding

  - Water Supply

    - Water Consumption

The maximum water consumption of the production line is about 2100m3/d. Among this  quantity, the production water is 1850m3/d, in which making up water for circulating cooling water system of 1500m3/d is included, water for living and virescence and so on is 250m3/d, and unpredictable water consumption is about 400m3/d.

A quantity of 160m3 is considered as firefighting water.

The circulating water volume is about 8000 m3/d.

    - Water Supply

Tube well water will be used for aforesaid purposes.

One 500m3 water pool will be built for circulating water, and one pump station to send circulating water to where needed.

| For firefighting water, one 300m3 water pool is needed to reserve the water, and this water could not be used for purposes other than firefighting. A firefighting pump station will be set up to supply water to where necessary Main Technical and Economic Indices No. | Description | SIndex |
| --- | --- | --- |
| 1 | Capacity |  |
|  | Clinker | 3300t/d |
|  | Cement, Thereinto: | 1,200,000 tons per year |
|  | 42.5# Portland cement (GB 175-99) | 500,000 tons per year |
|  | 42.5# OPC (GB 175-99) | 700,000 tons per year |
| 2 | Heat consumption | 740 kcal/kg.cl (3094KJ/kg.cl) |
| 3 | Power consumption | 98 kWh/t cement |
| 4 | Dust emission | 50mg/Nm3 (at outlet of EP) 50mg/Nm3 (at outlet of bag filter) |
| 5 | Total installed power | 25,600kW |
| 6 | Daily water consumption | for production: 1850m3/d for making up water: 1500m3/d for living in the plant: 250m3/d |