TABLE 3   TECHNICAL SPECIFICATION OF

JOINTED CYCLE STEAM TURBINES                       3-1

| No. | Designation & code Item Unit |  | *47MW Condensing turbine | *50MW Condensing turbine | *53.5MW Condensing turbine | *58MW Condensing turbine | *59MW Condensing turbine |  | **70MW Condensing turbine |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Code |  | 106# | 118# | 116# |  |  |  |  |  |  |
| 2 | Model |  | N47-3.95/473 | N50-4.37/475 | N53.5-4.70/500 | N58-4.56/0.266 /500/140 | N59-4.9/0.65/505 /274 |  | N68-65/0.65/505/252 |  |  |
| 3 | Type |  | Impulse 1-press.1-cylinder 1-exhaust | Impulse 1-press.1-cylinder 1-exhaust | Impulse 1-press.1-cylinder 1-exhaust | Impulse 2-press.1-cylinder 1-exhaust | Impulse 2-press.1-cylinder 1-exhaust |  | Impulse 3-press.1-cylinder 1-exhaust |  |  |
| 4 | Rated output | MW | 47 | 50 | 50 | 58.4 | 59.4 |  | 67.8 |  |  |
| 5 | Max.output | MW | 56.6 | 52 | 52 | 64 | 64 |  | 70.0 |  |  |
| 6 | Speed | r/min | 3000 | 3000 | 3000 | 3000 | 3000 |  | 3000 |  |  |
| 7 | Steam pres.before stop valve | Mpa | 3.95 | 4.37 | 4.70 | 4.56 | 4.9 |  | 6.5 |  |  |
| 8 | Steam temp.before stop valve | ℃ | 473 | 475 | 500 | 500 | 505 |  | 505 |  |  |
| 9 | Rated steam flow | t/h | 171 | 187 | 185 | 185 | 174 |  | 206 |  |  |
| 10 | Max steam flow | t/h | 203 | 189 | 208 | 195 | 186 |  | 215 |  |  |
| 11 | The second steam press. | MPa |  |  |  | 0.266 | 0.65 |  | 0.65 |  |  |
| 12 | The second steam temp. | ℃ |  |  |  | 140 | 274 |  | 256 |  |  |
| 13 | The rated second steam flow | t/h |  |  |  | 46 | 32 |  | 42 |  |  |
| 14 | The max second steam flow | t/h |  |  |  | 50 | 38 |  | 49 |  |  |
| 15 | Exhaust press (extract/condens) | MPa | 0.01 | 0.0101 | 0.0069 | 0.0065 |  | 0.0085 |  | 0.0085 |  |
| 16 | No. of stages |  | 18 | 18 | 18 | 18 |  | 18 |  | 19 |  |
| 17 | No. of heaters |  |  |  |  |  |  |  | IJD |  |  |
| 18 | Cooling of water temp | ℃ | 33 | 33 | 25 | 24 | 30 |  | 30 |  |  |
| 19 | Feed water temp | ℃ | 46 | 46 | 41 | 41 | 44 |  | 74 |  |  |
| 20 | Heat rate (extract/condens) | KJ/kW.h | 11615 | 11888 | 11348 | 12357 | 11604 |  | 10895 |  |  |
| 21 | Steam rate (extract/condens) | Kg/kW.h | 3.64 | 3.66 | 3.46 | 3.955 | 3.468 |  | 3.657 |  |  |
| 22 | Height of last vane | m2 | 665 | 665 | 665 | 710 | 668 |  | 800 |  |  |
| 23 | Overall size (LxWxH) | m | 8.6x6.9x2.6 | 8.6x6.9x2.6 | 8.6x6.9x2.6 | 8.6x7.0x2.9 | 8.9x7.0x2.9 |  | 9.3x7.6x3.5 |  |  |
| 24 | Turbine wt | t | 123 | 123 | 123 | 138 | 138 |  | 150 |  |  |
| 25 | Cooling surface of condenser | M2 | 5500 | 5500 | 5500 | 6000 | 560 |  | 6000 |  |  |
|  |  |  |  |  |  |  | Heavy oil |  | Heavy oil |  |  |
| Remarks |  | *To choose the about GE9E type burning machine, one driving one; or GE3000 or 6000 type burning machine, three driving one or four driving one **to choose SIEMENS v94.2 type burning machine, one driving one. |  |  |  |  |  |  |  |  |  |