TABLE 4  TECHNICAL SPECIFICATION OF

INDUSTRIAL STEAM TURBINES                 4-1

| No. | Designation & code Item Unit |  | 2.2MW Industrial steam turbine | 3MW Industrial steam turbine |  | 32MW Industrial steam turbine |  | 3.1MW Industrial steam turbine | 3.6MW Industrial steam turbine |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Code |  | 302# | 301# |  | 305# |  | 304# | 307# |
| 2 | Model |  | BZ31/31/04 | BZ55/55/04-A |  | BZ55/55/04-C |  | BZ61/55/04 | NGZ84.6/83.5/06 |
| 3 | Type |  | Back-press 1-cylinder | Back-press 1-cylinder |  | Back-press 1-cylinder |  | Back-press 1-cylinder | Impulse Condensing |
| 4 | Rated output | MW | 2.2 | 3 |  | 3 |  | 3.1 | 3.6 |
| 5 | Max.output | MW | 3.636 | 3 |  | 3 |  | 3.8 | 5.3 |
| 6 | Rated speed | r/min | 10962 | 5600 |  | 6352 |  | 5300 | 5400 |
| 7 | Max.speed | r/min | 11602 | 5600 |  | 6670 |  | 5535 | 5900 |
| 8 | Steam pres.before stop valve | Mpa | 3.432 | 3.432 |  | 3.432 |  | 3.433 | 0.788 |
| 9 | Steam temp.before stop valve | ℃ | 435±10 | 435 |  | 435 |  | 435+10 -5 | 335 |
| 10 | Rated steam flow | t/h | 35 | 55.75 |  | 50.04 |  | 50.29 | 20.2 |
| 11 | Exhaust press (extract/condense) | MPa | 1.079 | 1.275 |  | 1.079 |  | 1.079 | 0.008 |
| 12 | No. of stages |  | 4 | 4 |  | 4 |  | 4 | 1+5 |
| 13 | No. of heaters |  |  |  |  |  |  |  |  |
| 14 | Cooling of water temp | ℃ |  | 20 |  | 20 |  |  | 20 |
| 15 | Feed water temp | ℃ |  | 150 | 150 |  |  |  | 150 |
| 16 | Steam rate (extract/condense) | Kg/kw.h | 15.78 | 18.52 | 17 |  | 16.26 |  | 5.587 |
| 17 | Control model |  | Throttling control | Throttling control |  | Throttling control |  | Nozzle control | Nozzle control |
| 18 | Max lifting wt During erection | t | 8 | 6 |  | 6 |  | 6 | 16 |
| 19 | Overall size (LxWxH) | m | 3.23x1.29x1.21 | 3.38x1.3x2.3 |  | 3.38x1.3x2.3 |  | 4.0x2.4x2.0 | 3.3x2.5x3.2 |
| 20 | Turbine wt. | t | 12.6 | 17.73 |  | 22.66 |  | 19 | 30 |
| 21 | Cooling surface Of condenser | M2 |  |  |  |  |  |  | In common use with main condenser |
| 22 | Use |  | Driving compressor | Driving compressor & generator |  | Driving compressor & generator |  | Driving compressor & generator | Driving feed water pump |
| Remarks: |  |  |  |  |  |  |  |  |  |

TABLE 4  TECHNICAL SPECIFICATION OF

INDUSTRIAL STEAM TURBINES                 4-2

| No. | Designation & code Item Unit |  | 2.2MW Industrial steam turbine | 3MW Industrial steam turbine |  | 32MW Industrial steam turbine |  | 3.1MW Industrial steam turbine |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Code |  | 303# | Sx15# |  | 306# |  | 80# |  |  |  |
| 2 | Model |  | BZ55/55/04-B | BZ113/114/09 |  | NGZ83.5/83.5/06 |  | BZ88/86/09 |  |  |  |
| 3 | Type |  | Back-press 1-cylinder | Impulse Condensing |  | Impulse Condensing |  | Impulse Condensing |  |  |  |
| 4 | Rated output | MW | 4 | 6 |  | 6.9 |  | 12 |  |  |  |
| 5 | Max.output | MW | 4.2 |  |  | 9 |  | 14 |  |  |  |
| 6 | Rated speed | r/min | 5725 | 3000 |  | 5900 |  | 5333 |  |  |  |
| 7 | Max.speed | r/min | 6132 | 3100 |  | 5900 |  | 5760 |  |  |  |
| 8 | Steam pres.before stop valve | Mpa | 3.432 | 3.432 |  | 0.756 |  | 3.432 |  |  |  |
| 9 | Steam temp.before stop valve | ℃ | 435 | 435 |  | 335 |  | 435 |  |  |  |
| 10 | Rated steam flow | t/h | 71.13 | 30 |  | 36.3 |  | 55 |  |  |  |
| 11 | Exhaust press (extract/condense) | MPa | 1.275 |  |  | 0.0064 |  | 0.0098 |  |  |  |
| 12 | No. of stages |  | 4 | 1+8 |  | 1+5 |  | 1+8 |  |  |  |
| 13 | No. of heaters |  |  | 1(HP heater) +1(deaerator) +1(LP heater) |  |  |  | 1(HP heater) +1(deaerator) +1(LP heater) |  |  |  |
| 14 | Cooling of water temp | ℃ |  | 27 |  | 20 |  | 30 |  |  |  |
| 15 | Feed water temp | ℃ |  |  | 165 |  |  |  | 150 |  |  |
| 16 | Steam rate (extract/condense) | Kg/kw.h | 17.63 |  | 5.02 |  | 5.217 |  | 4.58 |  |  |
| 17 | Control model |  | Throttling control | Nozzle control |  | Nozzle control |  | Nozzle control |  |  |  |
| 18 | Max lifting wt During erection | t | 6 |  |  | 16 |  | 13 |  |  |  |
| 19 | Overall size (LxWxH) | m | 3.67x1.3x2.28 | 4.69x2.6x2.3 |  | 3.3x2.5x3.2 |  | 4.9x3.16x3.1 |  |  |  |
| 20 | Turbine wt. | t | 19.42 | 25.00 |  | 30.00 |  | 46.00 |  |  |  |
| 21 | Cooling surface Of condenser | M2 |  | 560 |  | In common use with main condenser |  | 900 |  |  |  |
| 22 | Use |  | Driving compressor | Driving compressor |  | Driving feed water Pump |  | Driving compressor |  |  |  |
| Remarks: |  |  |  |  |  |  |  |  |  |  |  |